###### Description of Requirements Document Format

Each row contains one requirement or closely-related set of requirements regarding pre-startup safety review.

The data contained in each column includes:

Column 1 –	reference to the box number on the associated process description in PS-8.1-PRD that applies to the requirement listed in Column 2

Column 2 –	written description of the requirement

Column 3 –	basis for the requirement or references for additional information

Columns 4 to 7 –	cells that indicate if the requirement listed in Column 2 applies to the various risk levels and regulated processes as defined in PS-1.2-MAP and related GIPSM documents

Column 8 –	notes on other information related to the requirement in Column 2

###### Instructions for Using This Requirements Document

- Select from columns 4 to 7 (according to coverage level [SCO, High Risk, or Lower Risk] and applicable regulations) for the process under consideration.

- Follow the selected column(s) down the page.  A cell containing an “X” indicates that the specific requirement stated in Column #2 (Requirements) applies to the particular process.  A cell containing a number indicates that the requirement may not apply if certain conditions are met.  An associated footnote describing these conditions will appear below the chart.

- Regulatory and Lilly Policy citations appear in Column #3 (References).

- The information found in the PS-8.1-PRD – Pre-Startup Safety Review associated with box numbers listed in Column 1 are also required actions.

###### Notes

- American Chemistry Council (formerly the Chemical Manufacturers Association) Responsible Care® Process Safety Code Management Practice #13 shall be implemented through implementation of the respective requirements shown on this chart and related maps and process descriptions for Safety Critical Operations, High Risk Processes, and Lower Risk Processes.

| 1Box No. | 2Requirements | 3References | 4Safety Critical Opns | 5High Risk Processes | 6Lower Risk Processes | 7Regulated Processes |  |  |  |  | 8Notes and Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | USA PSM/112(r) | GB COMAH | IRE Seveso II |  |  |  |
|  | PRESTARTUP SAFETY REVIEW REQUIREMENTS | 29 CFR 1910.119(i) 40 CFR 68.77 |  |  |  |  |  |  |  |  |  |
| 1 to 8 | [Conduct] Safety reviews on all new and modified facilities during design and prior to startup. | American Chemical Council Process Safety Code Management Practice 13 | X | X |  |  |  |  |  |  |  |
| 1 to 11 | Adopt and implement of procedures and instructions for safe operation, including maintenance of plant, processes, equipment, and temporary stoppages | Seveso II Articles 7 and 9 Annex III (c), part iii |  |  |  |  | X | X |  |  |  |
| 1 to 11 | Perform a pre-startup safety review for new facilities and for modified facilities when the modification is significant enough to required a change to the process safety information. For processes covered by EPA 112r 40 CFR 68, maintain completed PSSR checklists and supporting documentation of new and modified facilities for five years. | 29 CFR 1910.119(i)(1) 40 CFR 68.77(a) # 40 CFR 68.200 | X | X | X | X |  |  |  |  |  |
| 2, 3 or 4, 5 | The pre-startup safety review shall confirm that prior to the introduction of highly hazardous chemicals to a process: | 29 CFR 1910.119(i)(2) 40 CFR 68.77(b) |  |  |  |  |  |  |  |  | See GIPSM maps and related documents: PS-5.1-MAP – Operating Procedures PS-6.1 Training PS-11.1-MAP – Safe Work Practices PS-9.1-MAP – Mechanical Integrity PS-12.1-MAP – Emergency Planning and Response |
|  | - Construction and equipment is in accordance with design specifications; | 29 CFR 1910.119(i)(2)(i) 40 CFR 68.77(b)(1) | X | X | X | X |  |  |  |  |  |
|  | - Safety, operating maintenance, and emergency procedures are in place and are adequate; | 29 CFR 1910.119(i)(2)(ii) 40 CFR 68.77(b)(2) | X | X | X | X |  |  |  |  |  |
|  | - For new facilities [stationary sources], a process hazard analysis has been performed and recommendations have been resolved or implemented before startup; and modified facilities meet the requirements contained in management of change, paragraph (l). | 29 CFR 1910.119(i)(2)(iii) 40 CFR 68.77(b)(3) | X | X |  | X |  |  |  |  |  |
|  | - Training of each employee involved in operating a process has been completed. | 29 CFR 1910.119(i)(2)(iv) 40 CFR 68.77(b)(4) | X | X | X | X |  |  |  |  |  |
| 13 to 21 | Each site shall develop and implement restart procedures and/or checklists necessary to verify that maintenance work is complete and in accordance with design specifications. | GISPM Requirement | X | X |  |  |  |  |  |  |  |
| 18 | Each site shall develop and implement restart checklists necessary to ensure that work is complete and that the process is operationally ready for restart. | GIPSM Requirement | X | X | X |  |  |  |  |  |  |
| 20 | See SCO Requirement No. 30 | PS-1.3-REQ | X |  |  |  |  |  |  |  |  |